Variante A

Q: I want to instantiate a library cell
A: Answer from @Matt Venn in other thread is to use this: set ::env(SYNTH_READ_BLACKBOX_LIB) 1

Q: Hot to use Openlane Interactive Mode
A: https://openlane-docs.readthedocs.io/en/rtd-develop/doc/advanced_readme.html

Q: Cell design rules?
A: https://github.com/nickson-jose/vsdstdcelldesign (fixed_bbox, port settings, unithd, LEFclass/source)

Q: FIXED_BBOX size?
A: for HD: X:0.460 Y:0.340
property FIXED_BBOX {0 0 N*0,92 8*0,68} weil scale=0.5

Q: P&R Grid?
A: grid 0.46um 0.34um 0.23um 0.17um

Q: Root changes?
A: in openlane/scripts/floorplan disable basic macro floorplanning, macros not available if synth file read as blackbox

Shape abutment?

Variante B

Gate-Level to GDSII with OpenLane

The usual openlane-flow is RTL to GDSII, but we have no RTL file in this example. Instead, we want to skip Synthesis and convert a Gate-Level-Design to GDSII without further optimization and altering of cells.

Image from https://github.com/The-OpenROAD-Project/OpenLane/raw/master/docs/_static/openlane.flow.1.png

To achieve this, we must skip RTL-synthesis manually and prevent the tool from altering our cells. This is a possible workflow:

New openlane designs are usually initialized with ./flow.tcl -init-design-config , but this does not work in the docker environment. Instead, copy one of the design templates

- from /foss/tools/openlane/2022.07/designs/<designname>

- to /designs/<project-name>/openlane/<yournewdesignname>

Write your Verilog HDL-file in the format as shown in the image below. You can find names for the  standard cells in https://antmicro-skywater-pdk-docs.readthedocs.io and the PDK Verilog-library at /foss/pdk/sky130A/libs.ref/sky130_fd_sc_hd/verilog/sky130_fd_sc_hd.v

Change the config /designs/<project-name>/openlane/<yourdesignname>/config.tcl

  - Links to your Verilog-file

  - Name of your project

  - Prevent the router and placer from altering your cells by specifying

    - set ::env(PL_RESIZER_TIMING_OPTIMIZATIONS) {0}

    - set ::env(PL_RESIZER_DESIGN_OPTIMIZATIONS) {0}

    - set ::env(GLB_RESIZER_TIMING_OPTIMIZATIONS) {0}

  - Optional: set your pin order and fanout orientations with

    - set ::env(FP_PIN_ORDER_CFG) $::env(DESIGN_DIR)/pin_order.cfg

start a terminal at /designs/<project-name>/openlane/

run flow-tcl -interactive -design <yourdesignname> -tag <run-name>

type package require openlane, console should return 0.9

Before you start synthesis, copy your already synthesized file into 
..openlane/<yourdesignname>/runs/<run-name>/results/synthesis

Type run_synthesis in the console, synthesizer should recognize there is already a file (which is exactly what we want)

Type run_floorplan

Type run_placement

Type run_routing

Type run_magic

Result in runs/<run-name>/results/signoff/<project-name>.mag

…which is using the exact cell-type specified in our gate-level-design.

Appendix A: list of interactive openlane commands

https://github.com/The-OpenROAD-Project/OpenLane/blob/master/docs/source/advanced_readme.md

Appendix B: Configuration file of a CLKDLYBUF_4S50_2 combinatorial Delay-cell

Folder structure:

/foss/designs/SKY130_SAR-ADC

-/verilog

-/adc_edge_detect_circuit/ adc_edge_detect_circuit.v

-/openlane

-/adc_edge_detect_circuit/

-/runs/

-/config.tcl

-/pin_order.cfg

Contents of config.tcl:

set ::env(DESIGN_NAME) "adc_edge_detect_circuit"

set filename $::env(DESIGN_DIR)/$::env(PDK)_$::env(STD_CELL_LIBRARY)_config.tcl

if { [file exists $filename] == 1} {

source $filename

}

set ::env(VERILOG_FILES) "/foss/designs/SKY130_SAR-ADC/verilog/adc_edge_detect_circuit/adc_edge_detect_circuit.v"

# set ::env(CLOCK_PERIOD) "10.000"

set ::env(CLOCK_PORT) "clk"

set ::env(CLOCK_NET) $::env(CLOCK_PORT)

# Floorplanning

set ::env(FP_SIZING) "relative"

set ::env(FP_ASPECT_RATIO) {1}

set ::env(FP_CORE_UTIL) {75}

set ::env(FP_PDN_VPITCH) {100}

set ::env(FP_PDN_HPITCH) {100}

set ::env(FP_PIN_ORDER_CFG) $::env(DESIGN_DIR)/pin_order.cfg

# Placement

set ::env(PL_TARGET_DENSITY) {0.80}

# set ::env(PL_TIME_DRIVEN) {0}

set ::env(PL_RESIZER_TIMING_OPTIMIZATIONS) {0}

set ::env(PL_RESIZER_DESIGN_OPTIMIZATIONS) {0}

#Clocktreesynthesis - unused

# set ::env(CLOCK_TREE_SYNTH) {0}

# set ::env(FILL_INSERTION) {0}

# Router

set ::env(GLB_RESIZER_TIMING_OPTIMIZATIONS) {0}

set ::env(CELL_PAD) {0}