Gate-Level to GDSII with OpenLane

The usual flow is RTL to GDSII, but here we have no RTL file, instead we want to convert a Gate-Level-Design to GDSII. We must skip RTL synthesis manually and prevent the tool from altering our cells. This is a possible workflow:

flow-tcl -init-design-config does not work, instead copy one of the design templates

- from /foss/tools/openlane/2022.07/designs/<designname>

- to /designs/<project-name>/openlane/<yourdesignname>

Write your verilog-file in the format as shown below. You can find names for the high-density standard cells in https://antmicro-skywater-pdk-docs.readthedocs.io and the local Verilog library in /foss/pdk/sky130A/libs.ref/sky130_fd_sc_hd/verilog/sky130_fd_sc_hd.v

Change the config /designs/<project-name>/openlane/<yourdesignname>/config.tcl

  - Links to your Verilog-files

  - Name of your project

  - Prevent the router and placer from altering your cells by specifying

    - set ::env(PL_RESIZER_TIMING_OPTIMIZATIONS) {0}

    - set ::env(PL_RESIZER_DESIGN_OPTIMIZATIONS) {0}

    - set ::env(GLB_RESIZER_TIMING_OPTIMIZATIONS) {0}

  - Also set your pin order and fanout orientations with

    - set ::env(FP_PIN_ORDER_CFG) $::env(DESIGN_DIR)/pin_order.cfg

start a terminal at /designs/<project-name>/openlane/

run flow-tcl -interactive -design <yourdesignname> -tag <run-name>

type package require openlane, console should return 0.9

Before you start synthesis, place your already synthesized file in 
..openlane/<yourdesignname>/runs/<run-name>/results/synthesis

- Type run_synthesis in the console, synthesizer should recognize there is already a file (which is exactly what we want)

- Type run_floorplan

- Type run_placement

- Type run_routing

- Type run_magic

Result:

Appendix A: Complete list of interactive openlane commands

https://openlane-docs.readthedocs.io/en/rtd-develop/doc/advanced_readme.html

Appendix B: Configuration file of a CLKDLYBUF_4S50_2 combinatorial Delay-cell

Folder structure:

/foss/designs/SKY130_SAR-ADC

-/verilog

-/adc_edge_detect_circuit/ adc_edge_detect_circuit.v

-/openlane

-/adc_edge_detect_circuit/

-/runs/

-/config.tcl

-/pin_order.cfg

Contents of config.tcl:

set ::env(DESIGN_NAME) "adc_edge_detect_circuit"

set filename $::env(DESIGN_DIR)/$::env(PDK)_$::env(STD_CELL_LIBRARY)_config.tcl

if { [file exists $filename] == 1} {

source $filename

}

set ::env(VERILOG_FILES) "/foss/designs/SKY130_SAR-ADC/verilog/adc_edge_detect_circuit/adc_edge_detect_circuit.v"

# set ::env(CLOCK_PERIOD) "10.000"

set ::env(CLOCK_PORT) "clk"

set ::env(CLOCK_NET) $::env(CLOCK_PORT)

# Floorplanning

set ::env(FP_SIZING) "relative"

set ::env(FP_ASPECT_RATIO) {1}

set ::env(FP_CORE_UTIL) {75}

set ::env(FP_PDN_VPITCH) {100}

set ::env(FP_PDN_HPITCH) {100}

set ::env(FP_PIN_ORDER_CFG) $::env(DESIGN_DIR)/pin_order.cfg

# Placement

set ::env(PL_TARGET_DENSITY) {0.80}

# set ::env(PL_TIME_DRIVEN) {0}

set ::env(PL_RESIZER_TIMING_OPTIMIZATIONS) {0}

set ::env(PL_RESIZER_DESIGN_OPTIMIZATIONS) {0}

#Clocktreesynthesis - unused

# set ::env(CLOCK_TREE_SYNTH) {0}

# set ::env(FILL_INSERTION) {0}

# Router

set ::env(GLB_RESIZER_TIMING_OPTIMIZATIONS) {0}

set ::env(CELL_PAD) {0}